Configuring BGP For

Multiple Interior Gateway Protocols

Rui Zhao, CCNA

Purpose:

The purpose of this lab is the implementation of BGP-4 that correctly interconnects different routing domains like OSPF, EIGRP, and IS-IS to provide a professional enterprise-level complex setup. With BGP, it will serve as an inter-domain glue that is optimized for faster convergence, policy control, and scalability for the growing enterprise network.

Background:

The BGP protocol was established in January 1989 between Len Bosack and Yakov Rekhter’s 12th IETF meeting in Austin, Texas. The first model of the design was marked on two napkins, making the infamous reference “Two-Napkin Protocol.” As the idea improved, the writing was transcribed into written paper, establishing itself as a legitimate protocol on the Internet since 1994.

A year later, series of RFC documentation documented BGP’s definition. RFC 1136 proposed the model for routing over the growing Internet. Then, RFC defined BGP-3 as the exterior gateway protocol used to exchange routing and reachability information between systems, obsoleting earlier versions (RFC 1105’s BGP-1 and RFC 1163 BGP-2). The current version, however, was published as RFC 1654 as BGP-4. Furthermore, the latest definition of BGP4 is RFC 4271.

RFC 4271 defines the Border Gateway Protocol Version 4 (BGP-4) as a complex inter-Autonomous system routing protocol that is utilized mainly to exchange information with other BGP systems. BGP is classified as a path-vector protocol that makes routing and forwarding decisions based on weight, assigned policies, and rulesets set by the administrator. BGP-4 is derived and evolved from its predecessors – BGP-1, BGP-2, BGP-3 – and now incorporates Classless Inter-Domain Routing (CIDR). This design supported prefix aggregation in order to both facilitate and improve scalability and efficiency of routing.

BGP uses Transmission Control Protocol (TCP, mainly used for error-checking and ordered, reliable packets/data delivered over different networks) on port 179 to establish sessions that are secure and reliable between peers and to manage the routing advertisement exchanges. BGP advertises IP prefixes and associated attributes in order to allow networks to have flexibility over configurations regarding complex routing policies, loop-free interdomain routing, and scalability for the user or enterprise’s Internet routing architecture.

The BGP features two distinct yet similar processes: Internal BGP (iBGP, used for peering between routers in the same network) and External BGP (eBGP, used for exchanging information between different autonomous systems on the Internet. Both iBGP and eBGP both use the same way to communicate messages but choose the best paths differently. The eBGP process’s administrative distance (AD) is 20 and does not require a full mesh topology. It’s also used as path for loop prevention, and its default peers are set with time to live (TTL) as the value of 1. The iBGP process’s administrative distance is on the other hand is 200 and requires a full mesh topology. iBGP uses BGP Split horizon for loop prevention, and its default peers are set with time to live as the value of 255.

The current BGP configuration can support and adapt to a variety of routing protocols such as Multi-Area OSPF, EIGRP, and IS-IS. The way the BGP functions with different routing protocols is primary through route redistribution, where BGP speakers can import routes learned from the different routing protocols. By importing one route, BGP transfers and exports to another routing protocol. Furthermore, this is expanded by implementable route maps and filters to control the policies and routes for better path selection, offering a flexible way to control your Internet protocol environment.

Since we will be demonstrating Multi-Area OSPF, EIGRP, and IS-IS routing protocols during this lab, we will discuss each of their different characteristics below.

The Multi-Area OSPF is a routing protocol that utilizes the Open Shortest Path First (OSPF) routing process. OSPF is a link-state routing protocol that finds best paths via calculating the best paths, offering faster divergence and scalability across any network. Multi-Area expands on this by segmenting OSPF onto different areas, further facilitating scalability and simplified network divergence into segments, perfect for industry-level enterprises.

The Enhanced Interior Gateway Routing Protocol (EIGRP) is a routing protocol that utilizes both distance-vector and link-state protocols in order to achieve an efficient, scalable routing process within many networks, especially for enterprise networks. EIGRP uses many different metrics – bandwidth, delay, load, reliability – to calculate the best path, all which can be modified and configured by network administrators. It’s also mostly Cisco Proprietary, though an open standard version exists for other venders. Simply, the protocol works by neighbor discovery, route exchanges, and calculates feasible distances/successor.

The Intermediate System-to-Intermediate System (IS-IS) is a powerful link-state Interior Gateway Protocol (IGP) that’s mainly used in large service providers and enterprise networks in order to facilitate efficient routing like OSPF. IS-IS calculates the shortest path using the infamous Dijkstra Algorithm, which is a greedy path-finding algorithm in graph theory to find total minimum total weight from one source to all destinations in the network. The IS-IS uses a hierarchy system: Level 1 (within areas) and Level 2 (backbone) for scalability.

Lab Commands:

| Device | G0/0/0 | G0/0/1 | Lo0 | Router-ID |
| --- | --- | --- | --- | --- |
| R1 | N/A | 192.168.1.1/24 | 10.10.1.1/24 | 1.1.1.1 |
|  | N/A | 2001:DB8:ACAD:1::1/64 | 2001:DB8:ACAD:11::1/64 | 1.1.1.1 |
| R2 | 192.168.1.2/24 | 192.168.2.1/24 | 10.10.2.1/24 | 2.2.2.2 |
|  | 2001:DB8:ACAD:1::2/64 | 2001:DB8:ACAD:2::1/64 | 2001:DB8:ACAD:12::1/64 | 2.2.2.2 |
| R3 | 192.168.2.2/24 | 192.168.3.1/24 | 10.10.3.1/24 | 3.3.3.3 |
|  | 2001:DB8:ACAD:2::2/64 | 2001:DB8:ACAD:3::1/64 | 2001:DB8:ACAD:13::1/64 | 3.3.3.3 |
| R4 | 192.168.3.2/24 | 192.168.4.1/24 | 10.10.4.1/24 | 4.4.4.4 |
|  | 2001:DB8:ACAD:3::2/64 | 2001:DB8:ACAD:4::1/64 | 2001:DB8:ACAD:14::1/64 | 4.4.4.4 |
| R5 | 192.168.4.2/24 | 192.168.5.1/24 | 10.10.5.1/24 | 49.0001.0000 .0000.000a.00 |
|  | 2001:DB8:ACAD:4::2/64 | 2001:DB8:ACAD:5::1/64 | 2001:DB8:ACAD:15::1/64 | 49.0001.0000 .0000.000a.00 |
| R6 | 192.168.5.2/24 | N/A | 10.10.6.1/24 | 49.0001.0000 .0000.000b.00 |
|  | 2001:DB8:ACAD:5::2/64 | N/A | 2001:DB8:ACAD:16::1/64 | 49.0001.0000 .0000.000b.00 |

Configuring the address table (All routers):

Set up all the IPv4 & IPv6 for each router according to the topology.

IPv4:

R1(config) # int g0/0/1

R1(config-if) # ip address 192.168.1.1 255.255.255.0

R1(config-if) # ipv6 address FE80::1 link-local

R1(config-if) # ipv6 address 2001:DB8:ACAD:1::1/64

R1(config-if) # no shut

R1(config-if) # int lo0

R1(config-if) # ip address 10.10.1.1 255.255.255.0

R1(config-if) # ipv6 address 2001:DB8:ACAD:11::1/64

R1(config-if) # no shut

R1(config-if) # exit

R1(config) # ipv6 unicast-routing

R2(config) # int g0/0/0

R2(config-if) # ip address 192.168.1.2 255.255.255.0

R2(config-if) # ipv6 address 2001:DB8:ACAD:1::2/64

R2(config-if) # ipv6 address FE80::2 link-local

R2(config-if) # no shut

R2(config-if) # int g0/0/1

R2(config-if) # ip address 192.168.2.1 255.255.255.0

R2(config-if) # ipv6 address 2001:DB8:ACAD:2::1/64

R2(config-if) # ipv6 address FE80::1 link-local

R2(config-if) # no shut

R2(config-if) # int lo0

R2(config-if) # ip address 10.10.2.1 255.255.255.0

R2(config-if) # ipv6 address 2001:DB8:ACAD:12::1/64

R2(config-if) # no shut

R2(config-if) # exit

R2(config) # ipv6 unicast-routing

R3(config) # int g0/0/0

R3(config-if) # ip address 192.168.2.2 255.255.255.0

R3(config-if) # ipv6 address 2001:DB8:ACAD:2::2/64

R3(config-if) # ipv6 address FE80::2 link-local

R3(config-if) # no shut

R3(config-if) # int g0/0/1

R3(config-if) # ip address 192.168.3.1 255.255.255.0

R3(config-if) # ipv6 address 2001:DB8:ACAD:3::1/64

R3(config-if) # ip ipv6 address FE80::1 link-local

R3(config-if) # ipv6 enable

R3(config-if) # no shut

R3(config-if) # int lo0

R3(config-if) # ip address 10.10.3.1 255.255.255.0

R3(config-if) # ipv6 address 2001:DB8:ACAD:13::1/64

R3(config-if) # ipv6 enable

R3(config-if) # no shut

R3(config-if) # exit

R3(config) # ipv6 unicast-routing

R4(config) # int g0/0/0

R4(config-if) # ip address 192.168.3.2 255.255.255.0

R4(config-if) # ipv6 address 2001:DB8:ACAD:3::2/64

R4(config-if) # ipv6 address FE80::2 link-local

R4(config-if) # ipv6 enable

R4(config-if) # no shut

R4(config-if) # int g0/0/1

R4(config-if) # ip address 192.168.4.1 255.255.255.0

R4(config-if) # ipv6 address 2001:DB8:ACAD:4::1/64

R4(config-if) # ipv6 address FE80::1 link-local

R4(config-if) # no shut

R4(config-if) # int lo0

R4(config-if) # ip address 10.10.4.1 255.255.255.0

R4(config-if) # ipv6 address 2001:DB8:ACAD:14::1/64

R4(config-if) # ipv6 enable

R4(config-if) # no shut

R4(config-if) # exit

R4(config) # ipv6 unicast-routing

R5(config) # int g0/0/0

R5(config-if) # ip address 192.168.4.2 255.255.255.0

R5(config-if) # ipv6 address 2001:DB8:ACAD:4::2/64

R5(config-if) # ipv6 address FE80::2 link-local

R5(config-if) # no shut

R5(config-if) # int g0/0/1

R5(config-if) # ip address 192.168.5.1 255.255.255.0

R5(config-if) # ipv6 address 2001:DB8:ACAD:5::1/64

R5(config-if) # ipv6 address FE80::1 link-local

R5(config-if) # no shut

R5(config-if) # int lo0

R5(config-if) # ip address 10.10.5.1 255.255.255.0

R5(config-if) # ipv6 address 2001:DB8:ACAD:15::1/64

R5(config-if) # ipv6 enable

R5(config-if) # no shut

R5(config-if) # exit

R5(config) # ipv6 unicast-routing

R6(config) # int g0/0/0

R6(config-if) # ip address 192.168.5.2 255.255.255.0

R6(config-if) # ipv6 address 2001:DB8:ACAD:5::2/64

R6(config-if) # ipv6 address FE80::2 link-local

R6(config-if) # no shut

R6(config-if) # int lo0

R6(config-if) # ip address 10.10.6.1 255.255.255.0

R6(config-if) # ipv6 address 2001:DB8:ACAD:16::1/64

R6(config-if) # no shut

R6(config-if) # exit

R6(config) # ipv6 unicast-routing

Configuring OSPF (Routers 1 & 2):

First, configure the routers’ IPv4 router-id for OSPF for router identification during discovery process and associate the network.

R1(config) # router ospf 1

R1(config-router) # router-id 1.1.1.1

R1(config-router) # network 192.168.1.0 0.0.0.255 area 0

R1(config-router) # exit

R2(config) # router ospf 1

R2(config-router) # router-id 2.2.2.2

R2(config-router) # network 192.168.1.0 0.0.0.255 area 0

R2(config-router) # exit

Then, configure the routers’ IPv6 router-id for OSPF for router identification during discovery process and associate the network.

R1(config) # ipv6 router ospf 1

R1(config-router) # router-id 1.1.1.1

R1(config-router) # exit

R2(config) # ipv6 router ospf 1

R2(config-router) # router-id 2.2.2.2

R2(config-router) # exit

After that, proceed to incorporate the topology IPv4/IPv6 addresses and associate the IPv4/IPv6 OSPF with the interfaces. We’ll also be configuring loopback interfaces as a tool to aid verification of BGP as it makes BGP redistribution visible on the routing table.

R1(config) # int g0/0/1

R1(config-if) # ip ospf 1 area 0

R1(config-if) # ipv6 ospf 1 area 0

R1(config-if) # no shut

R1(config-if) # exit

R1(config) # int lo0

R1(config-if) # ip ospf 1 area 2

R1(config-if) # ipv6 ospf 1 area 2

R1(config-if) # no shut

R1# clear ip ospf process

Reset selected OSPFv2 processes? [no]:y

R1# clear ipv6 ospf process

Reset selected OPSFv3 processes? [no]:y

R2(config) # int g0/0/0

R2(config-if) # ip ospf 1 area 0

R2(config-if) # ipv6 ospf 1 area 0

R2(config-if) # no shut

R2(config-if) # exit

R2(config) # int g0/0/1

R2(config-if) # no shut

R2(config-if) # exit

R2(config) # int lo0

R2(config-if) # ip ospf 1 area 1

R2(config-if) # ipv6 ospf 1 area 1

R2(config-if) # no shut

R2# clear ip ospf process

Reset selected OSPFv2 processes? [no]:y

R2# clear ipv6 ospf process

Reset selected OPSFv3 processes? [no]:y

Configuring EIGRP (Routers 3 & 4)

First, configure the router’s IPv4 EIGRP router-id and set up its default metrics (adjustable).  This will allow router identification through the EIGRPv4 database. Wildcard-masks are also optional but supported.

R3(config) # ip router eigrp 100

R3(config-router) # eigrp router-id 3.3.3.3

R3(config-router) # default-metric 10000 100 255 1 1500

R3(config-router) # network 192.168.3.0 0.0.0.255

R3(config-router) # network 10.0.0.0 0.0.0.255

R3(config-router) # exit

R4(config) # ip router eigrp 100

R4(config-router) # eigrp router-id 4.4.4.4

R4(config-router) # default-metric 10000 100 255 1 1500

R4(config-router) # network 192.168.3.0 0.0.0.255

R4(config-router) # network 192.168.4.0 0.0.0.255

R4(config-router) # network 10.0.0.0 0.0.0.255

R4(config-router) # exit

Then, configure the router’s IPv6 EIGRP router-id and set up its default metrics (adjustable).  This will allow router identification through the EIGRPv6 database. Wildcard-masks are also optional but supported.

R3(config) # ipv6 router eigrp 100

R3(config-router) # eigrp router-id 3.3.3.3

R3(config-router) # default-metric 10000 100 255 1 1500

R3(config-router) # exit

R3(config) # int g0/0/1

R3(config-if) # ipv6 eigrp 100

R3(config-if) # exit

R3(config) # int lo0

R3(config-if) # ipv6 eigrp 100

R3(config-if) # exit

R4(config) # ipv6 router eigrp 100

R4(config-router) # eigrp router-id 4.4.4.4

R4(config-router) # default-metric 10000 100 255 1 1500

R4(config-router) # exit

R4(config) # int g0/0/0

R4(config-if) # ipv6 eigrp 100

R4(config-if) # exit

R4(config) # int lo0

R4(config-if) # ipv6 eigrp 100

R4(config-if) # exit

Configuring IS-IS (Routers 5 & 6)

First, configure the router’s IPv4 & IPv6 IS-IS router with IS-IS net ID to establish control plane communication and topology/router identification. Then, attach this onto the appropriate interfaces.

R5(config) # router isis 1

R5(config-router) # net 49.0001.0000.0000.000a.00

R5(config-router) # exit

R6(config) # router isis 1

R6(config-router) # net 49.0001.0000.0000.000b.00

R6(config-router) # exit

Then, associate the router’s IPv4 & IPv6 IS-IS router configurations to the appropriate interfaces in order to activate the IS-IS routing protocol.

R5(config) # int g0/0/1

R5(config-if) # ip router isis 1

R5(config-if) # ipv6 router isis 1

R5(config-if) # exit

R5(config) # int lo0

R5(config-if) # ip router isis 1

R5(config-if) # ipv6 router isis 1

R5(config-if) # exit

R6(config) # int g0/0/0

R6(config-if) # ip router isis 1

R6(config-if) # ipv6 router isis 1

R6(config-if) # exit

R6(config) # int lo0

R6(config-if) # ip router isis 1

R6(config-if) # ipv6 router isis 1

R6(config-if) # exit

Redistributing and Neighbor Adjacencies with BGP

In order to really exchange routes between different routing protocols, BGP needs to act as a middleman to import their exchanges into BGP and export them into another routing protocol. This process not only enables communication but also integrates new networks and simplifies complex ones. We’ll segment each routing protocol with their own respective BGP links and IPv4/IPv6 counterparts.

IPv4: OSPF to BGP

R2(config) # router bgp 65001

R2(config-router) # address-family ipv4

R2(config-router-ad) # neighbor 192.168.2.2 remote-as 65002

R2(config-router-ad) # neighbor 192.168.2.2 activate

R2(config-router-ad) # redistribute ospf 1 include-connected

R2(config-router-ad) # end

R2(config) # router ospf 1

R2(config-router) # redistribute bgp 65001 include-connected

R2(config-router) # end

IPv6: OSPF to BGP

R2(config) # router bgp 65001

R2(config-router) # address-family ipv6

R2(config-router-ad) # neighbor 2001:DB8:ACAD:2::2 remote-as 65002

R2(config-router-ad) # neighbor 2001:DB8:ACAD:2::2 activate

R2(config-router-ad) # redistribute ospfv3 1 include-connected

R2(config-router-ad) # end

R2(config) # router ospfv3 1

R2(config-router) # redistribute bgp 65001 include-connected

R2(config-router) # end

IPv4: EIGRP to BGP

R3(config) # router bgp 65002

R3(config-router) # address-family ipv4

R3(config-router-ad) # neighbor 192.168.2.1 remote-as 65001

R3config-router-ad) # neighbor 192.168.2.1 activate

R3(config-router-ad) # neighbor 192.168.4.2 remote-as 65003

R3(config-router-ad) # neighbor 192.168.4.2 activate

R3(config-router-ad) # redistribute eigrp 100 include-connected

R3(config-router-ad) # end

R3(config) # router eigrp 100

R3(config-router) # redistribute bgp 65002 include-connected metric 10000 100 255 1 1500

R3(config-router) # end

R4(config) # router bgp 65002

R4(config-router) # address-family ipv4

R4(config-router-ad) # neighbor 192.168.2.1 remote-as 65001

R4(config-router-ad) # neighbor 192.168.2.1 activate

R4(config-router-ad) # neighbor 192.168.4.2 remote-as 65003

R4(config-router-ad) # neighbor 192.168.4.2 activate

R4(config-router-ad) # redistribute eigrp 100 include-connected

R4(config-router-ad) # end

R4(config) # router eigrp 100

R4(config-router) # redistribute bgp 65002 include-connected metric 100000 100 255 1 1500

R4(config-router) # end

IPv6: EIGRP to BGP

R3(config) # router bgp 65002

R3(config-router) # address-family ipv6

R3(config-router-ad) # neighbor 2001:DB8:ACAD:2::1 remote-as 65001

R3(config-router-ad) # neighbor 2001:DB8:ACAD:2::1 activate

R3(config-router-ad) # redistribute eigrp 100 include-connected

R3(config-router-ad) # end

R3(config) # router eigrp 100

R3(config-router) # redistribute bgp 65002 include-connected metric 10000 100 255 1 1500

R3(config-router) # end

R4(config) # router bgp 65002

R4(config-router) # address-family ipv6

R4(config-router-ad) # neighbor 2001:DB8:ACAD:2::1 remote-as 65001

R4(config-router-ad) # neighbor 2001:DB8:ACAD:2::1 activate

R4(config-router-ad) # neighbor neighbor 2001:DB8:ACAD:4::2 remote-as 65003

R4(config-router-ad) # neighbor 2001:DB8:ACAD:4::2 activate

R4(config-router-ad) # redistribute eigrp 100 include-connected

R4(config-router-ad) # end

R4(config) # ipv6 router eigrp 100

R4(config-router) # redistribute bgp 65002 include-connected metric 10000 100 255 1 1500

R4(config-router) # end

IPv4: IS-IS to BGP

R5(config) # router bgp 65003

R5(config-router) # address-family ipv4

R5(config-router-ad) # neighbor 192.168.4.1 remote-as 65002

R5(config-router-ad) # neighbor 192.168.4.1 activate

R5(config-router-ad) # redistribute connected

R5(config-router-ad) # redistribute isis 1 level-1

R5(config-router-ad) # end

R5(config) # router isis 1

R5(config-router) # address-family ipv4

R5(config-router-ad) # redistribute bgp 65003 include-connected

R5(config-router-ad) # end

IPv6: IS-IS to BGP

R5(config) # router bgp 65003

R5(config-router) # address-family ipv6

R5(config-router-ad) # neighbor 2001:DB8:ACAD:4::1 remote-as 65002

R5(config-router-ad) # neighbor 2001:DB8:ACAD:4::1 activate

R5(config-router-ad) # redistribute isis 1 level-1-2 include-connected

R5(config-router-ad) # redistribute isis level-1

R5(config-router-ad) # end

R5(config) # router isis 1

R5(config-router) # address-family ipv6

R5(config-router-ad) # redistribute bgp 65003 include-connected

R5(config-router-ad) # end

Configuring Weights:

Weights, in the deep context of BGP, are Cisco Proprietary property used to influence path selection for flexible adjustments. Routes with higher values are preferred over routes with lower ones, which are adjustable. For this example, we will be configuring the weights on R2 where we set our neighbor’s (R3) weights to 500 for both IPv4 and IPv6.

R2(config) # router bgp 65001

R2(config-router) # address-family ipv4

R2(config-router-ad) # neighbor 192.168.2.2 weight 500

R2(config-router-ad) # exit

R2(config-router) # address-family ipv6

R2(config-router-ad) # neighbor 2001:DB8:ACAD:2::2 weight 500

Configuring Route-Maps:

Route-Maps are powerful tools in the BGP context to tackle and implement complex routing policies that filter, modify, and facilitate over route control and how BGP attributes are manipulated. For this example, we will be configuring route-maps on R2.

R2(config) # ip prefix-list OSPF-IMPORT seq 5 permit 10.10.3.0/24

R2(config) # ip prefix-list OSPF-IMPORT seq 5 permit 10.10.4.0/24

R2(config) # ip prefix-list OSPF-IMPORT seq 5 permit 10.10.5.0/24

R2(config) # ip prefix-list OSPF-IMPORT seq 5 permit 10.10.6.0/24

R2(config) # ip prefix-list OSPF-IMPORT seq 5 permit 192.168.2.0/24

R2(config) # ip prefix-list OSPF-IMPORT seq 5 permit 192.168.3.0/24

R2(config) # ip prefix-list OSPF-IMPORT seq 5 permit 192.168.4.0/24

R2(config) # ip prefix-list OSPF-IMPORT seq 5 permit 192.168.5.0/24

R2(config) # route-map BGP-TO-OSPFv4 permit 10

R2(config-route-map) # match ip address prefix-list OSPF-IMPORT

R2(config-route-map) # exit

R2(config) # ipv6 prefix-list OSPF-IMPORT seq 5 permit 2001:DB8:ACAD:2::0/64

R2(config) # ipv6 prefix-list OSPF-IMPORT seq 5 permit 2001:DB8:ACAD:3::0/64

R2(config) # ipv6 prefix-list OSPF-IMPORT seq 5 permit 2001:DB8:ACAD:4::0/64

R2(config) # ipv6 prefix-list OSPF-IMPORT seq 5 permit 2001:DB8:ACAD:5::0/64

R2(config) # ipv6 prefix-list OSPF-IMPORT seq 5 permit 2001:DB8:ACAD:13::0/64

R2(config) # ipv6 prefix-list OSPF-IMPORT seq 5 permit 2001:DB8:ACAD:14::0/64

R2(config) # ipv6 prefix-list OSPF-IMPORT seq 5 permit 2001:DB8:ACAD:15::0/64

R2(config) # ipv6 prefix-list OSPF-IMPORT seq 5 permit 2001:DB8:ACAD:16::0/64

R2(config) # route-map BGP-TO-OSPFv4 permit 10

R2(config-route-map) # match ipv6 address prefix-list OSPF-IMPORT

R2(config-route-map) # exit

R2(config) # router bgp 65001

R2(config-router) # address-family ipv4

R2(config-router-ad) # neighbor 192.168.2.2 route-map BGP-TO-OSPFv4 in

R2(config-router-ad) # exit

R2(config-router) # address-family ipv6

R2(config-router-ad) # neighbor 2001:DB8:ACAD:2::2 route-map BGP-TO-OSPFv6 in

R2(config-router-ad) # end

Network Configuration

_________________________________________________________________________________________________________

Router 1:

Building configuration...

Current configuration : 4109 bytes

Last configuration change at 18:28:21 UTC Fri Oct 31 2025

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R1

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

crypto pki trustpoint TP-self-signed-2083466926

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-2083466926

revocation-check none

rsakeypair TP-self-signed-2083466926

crypto pki certificate chain TP-self-signed-2083466926

certificate self-signed 01

30820330 30820218 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 32303833 34363639 3236301E 170D3235 31303238 31373139

32385A17 0D333030 31303130 30303030 305A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D32 30383334

36363932 36308201 22300D06 092A8648 86F70D01 01010500 0382010F 00308201

0A028201 0100C1F3 80A6598E 775C4AFA 20ECF461 1625A61C 69FB81B9 5C7F83BD

FBCE1DDE 88818A7A 97586E0B D98CF05E AE1C99BB 119DB836 0840851F C06EBDE8

C828C00F 6C6FEF05 DDA8E50C 85509ADF 128721A0 D5727FEA 51DBDB3A 0FF73045

E8094027 EAA96833 8D039F32 FC6A0852 D65680F2 7E00F00A 1F9D472B A978D8B5

E131C586 00E4C4BA F812F1AB 176A1D84 AADA7DDC 78814619 B987D0B9 0A094737

D9EE3F27 2AA91992 A183D8C3 4608968B 825E59E5 14609B75 00BC86D6 BF0E1127

3C95170C 05396BCA 56FFBE79 541E25DD BAFFF121 698C61BE AF86A91A 33E968FC

CCACF465 5CE98C9F 307AB1EA ACFDA299 BB3FACA3 926A50EA 5CF5BDFE B2DCA221

6BB9E01D 634F0203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF

301F0603 551D2304 18301680 14FE7979 2514871F 5B66B8ED ADDB90B0 34DFD038

E4301D06 03551D0E 04160414 FE797925 14871F5B 66B8EDAD DB90B034 DFD038E4

300D0609 2A864886 F70D0101 05050003 82010100 C0527A57 D4235178 1F79C523

2D5B4F62 5C652F8A 00B1B7FE 2797989E E0131C88 ABFB3C0B 819E83D2 59EAF247

66417D65 819FB7D4 4B2C0A84 E309AAF8 7D2E76E0 90535066 C2D16CAB 95FAAE47

DBEEA1A1 F60CD1D2 74C58C9A E9911714 4BE2FA30 8F64F10E DDE149A2 8DE58A5D

0B7ED907 0A380E3E 46163DE8 6DB14285 4FD17428 074A5647 66589E0E 0084E26E

E20BB759 41B60820 4D345A25 1A24F51F 3C3AE03E E6112C9A A240BAD1 5D67CB95

00FB9C5C 5AD5A3CD 64DEE233 C429BD13 73274144 D805CB5E A1EE64C0 91C5A003

5056692C 8F9DCC05 16393F8C A98570A4 9B211465 53C086E3 6A01B375 7584C048

219CA3B3 FF8390EE 89A8036D 6F7EC37C 2D01157B

quit

license udi pid ISR4321/K9 sn FDO21281AAT

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 10.10.1.1 255.255.255.0

ip ospf 1 area 2

ipv6 address 2001:DB8:ACAD:11::1/64

ipv6 ospf 1 area 2

interface GigabitEthernet0/0/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/0/1

ip address 192.168.1.1 255.255.255.0

ip ospf 1 area 0

negotiation auto

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:1::1/64

ipv6 ospf 1 area 0

interface GigabitEthernet0/1/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/1/1

no ip address

shutdown

negotiation auto

interface Service-Engine0/2/0

no ip address

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router ospfv3 1

router-id 1.1.1.1

address-family ipv6 unicast

exit-address-family

router ospf 1

router-id 1.1.1.1

network 192.168.1.0 0.0.0.255 area 0

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

_________________________________________________________________________________________________________

Router 2:

Building configuration...

Current configuration : 6230 bytes

Last configuration change at 19:58:20 UTC Mon Nov 24 2025

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R2

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

crypto pki trustpoint TP-self-signed-702538597

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-702538597

revocation-check none

rsakeypair TP-self-signed-702538597

crypto pki certificate chain TP-self-signed-702538597

certificate self-signed 01

3082032E 30820216 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

30312E30 2C060355 04031325 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 37303235 33383539 37301E17 0D323531 31323431 39333735

345A170D 33303031 30313030 30303030 5A303031 2E302C06 03550403 1325494F

532D5365 6C662D53 69676E65 642D4365 72746966 69636174 652D3730 32353338

35393730 82012230 0D06092A 864886F7 0D010101 05000382 010F0030 82010A02

82010100 D94BC711 C8399F28 C1DEE40E A93F1A23 4013F6ED B5647124 E9C26CE5

F6499989 F7107F62 4F29CCB6 8DB91040 0E3BD571 140AF0C3 326A8438 13EECE4E

5947438B AAB97F60 239165BE 9B4FA509 F0A7A77D 903DB186 B4180BA3 3D288CBE

9022408E FEBB3639 84B3BD11 4EECF80C 2550FEFF 01DEE0F0 50DEB4B3 A86AFA47

495E7314 0919A2AB D35A3DB0 7365B2A3 46F60A79 73263287 769A964C 624939DD

AE519FCC 5EDB67FA 39689A36 E2813A89 F2AAA17C F2F8B4A0 2934C665 2C32A84F

75D8C467 C785D1E9 455C3ABD E675A8B7 5DA2A97D 8CD84E0E 5D198FFE 6181D305

097C74C4 2569A0EF CF22ECE8 76B85278 BD5D13AE ED75BA57 A87B3FFC 01FDE419

971DA247 02030100 01A35330 51300F06 03551D13 0101FF04 05300301 01FF301F

0603551D 23041830 16801436 35B79976 6377E827 18D059AF E41A35CC E55FE330

1D060355 1D0E0416 04143635 B7997663 77E82718 D059AFE4 1A35CCE5 5FE3300D

06092A86 4886F70D 01010505 00038201 0100230B 747CC558 16C85DEF 9AF5B4C1

202EA0B5 97BADD43 A5BA8A01 3165F557 33941939 AF2CF301 B05D7308 1C622174

C5CA91CF D71BAC7E 9C9E51D1 D1B0FB00 476A6598 DBC28269 89B2DE2D 8E5813C2

C4F09C26 DBCEDD30 BFCF7A56 E0727D11 9B39E62E 74F32F58 683D0C63 C3AA2F21

29E6AD43 F98671AC 0BC9FA93 C1D4E51A 2CC70F2F 0E26FAB0 C680ABA2 E9F46753

7B84EA6D DA3B8C79 756468FE 93C68B37 4CF8196F 884AE58E 0EC1CD75 A9718ED4

D8E9A211 5D1BD1AE D6A26FA2 B6B3A2AD A03789C8 B4FE0A73 C3CD7A67 CF983DB3

07C9EED3 E4466C73 004378AC 6EF83380 3B7F2929 FB57CAB0 8C958630 085A5210

7B6C2FFD 2C10F586 D13D04BA 8831B596 DF9D

quit

license udi pid ISR4321/K9 sn FDO21491LXF

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 10.10.2.1 255.255.255.0

ip ospf 1 area 1

ipv6 address 2001:DB8:ACAD:12::1/64

ipv6 ospf 1 area 1

interface GigabitEthernet0/0/0

ip address 192.168.1.2 255.255.255.0

ip ospf 1 area 0

negotiation auto

ipv6 address FE80::2 link-local

ipv6 address 2001:DB8:ACAD:1::2/64

ipv6 ospf 1 area 0

interface GigabitEthernet0/0/1

ip address 192.168.2.1 255.255.255.0

negotiation auto

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:2::1/64

interface Serial0/1/0

no ip address

shutdown

interface Serial0/1/1

no ip address

shutdown

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router ospfv3 1

router-id 2.2.2.2

address-family ipv4 unicast

exit-address-family

address-family ipv6 unicast

redistribute bgp 65001

exit-address-family

router ospf 1

router-id 2.2.2.2

redistribute connected subnets route-map BGP-TO-OSPFv4

redistribute bgp 65001 subnets route-map BGP-TO-OSPFv4

network 192.168.1.0 0.0.0.255 area 0

router bgp 65001

bgp log-neighbor-changes

no bgp default ipv4-unicast

neighbor 2001:DB8:ACAD:2::2 remote-as 65002

neighbor 192.168.2.2 remote-as 65002

address-family ipv4

network 192.168.2.0

redistribute connected

redistribute ospf 1

redistribute ospfv3 1

neighbor 192.168.2.2 activate

neighbor 192.168.2.2 weight 500

neighbor 192.168.2.2 route-map BGP-TO-OSPFv4 in

exit-address-family

address-family ipv6

redistribute connected

redistribute ospf 1 include-connected

neighbor 2001:DB8:ACAD:2::2 activate

neighbor 2001:DB8:ACAD:2::2 weight 500

neighbor 2001:DB8:ACAD:2::2 route-map BGP-TO-OSPFv6 in

exit-address-family

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

ip prefix-list OSPF-IMPORT seq 5 permit 10.10.3.0/24

ip prefix-list OSPF-IMPORT seq 10 permit 10.10.4.0/24

ip prefix-list OSPF-IMPORT seq 15 permit 10.10.5.0/24

ip prefix-list OSPF-IMPORT seq 20 permit 10.10.6.0/24

ip prefix-list OSPF-IMPORT seq 25 permit 192.168.2.0/24

ip prefix-list OSPF-IMPORT seq 30 permit 192.168.3.0/24

ip prefix-list OSPF-IMPORT seq 35 permit 192.168.4.0/24

ip prefix-list OSPF-IMPORT seq 40 permit 192.168.5.0/24

ipv6 prefix-list OSPF-IMPORT seq 5 permit 2001:DB8:ACAD:2::/64

ipv6 prefix-list OSPF-IMPORT seq 10 permit 2001:DB8:ACAD:3::/64

ipv6 prefix-list OSPF-IMPORT seq 15 permit 2001:DB8:ACAD:4::/64

ipv6 prefix-list OSPF-IMPORT seq 20 permit 2001:DB8:ACAD:5::/64

ipv6 prefix-list OSPF-IMPORT seq 25 permit 2001:DB8:ACAD:13::/64

ipv6 prefix-list OSPF-IMPORT seq 30 permit 2001:DB8:ACAD:14::/64

ipv6 prefix-list OSPF-IMPORT seq 35 permit 2001:DB8:ACAD:15::/64

ipv6 prefix-list OSPF-IMPORT seq 40 permit 2001:DB8:ACAD:16::/64

route-map BGP-TO-OSPFv6 permit 10

match ipv6 address prefix-list OSPF-IMPORT

set metric 10

set tag 100

set local-preference 120

route-map BPG-TO-OSPFv6 permit 10

route-map BGP-TO-OSPFv4 permit 10

match ip address prefix-list OSPF-IMPORT

set metric 10

set tag 100

set local-preference 120

route-map BPG-TO-OSPFv4 permit 10

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

_________________________________________________________________________________________________________

Router 3:

Building configuration...

Current configuration : 2519 bytes

Last configuration change at 19:09:22 UTC Tue Nov 4 2025

version 15.5

service timestamps debug datetime msec

service timestamps log datetime msec

no platform punt-keepalive disable-kernel-core

hostname R3

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

ipv6 unicast-routing

subscriber templating

vtp domain cisco

vtp mode transparent

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FLM240607Q1

spanning-tree extend system-id

redundancy

mode none

vlan internal allocation policy ascending

interface Loopback0

ip address 10.10.3.1 255.255.255.0

ipv6 address 2001:DB8:ACAD:13::1/64

ipv6 enable

ipv6 eigrp 100

interface GigabitEthernet0/0/0

ip address 192.168.2.2 255.255.255.0

negotiation auto

ipv6 address FE80::2 link-local

ipv6 address 2001:DB8:ACAD:2::2/64

interface GigabitEthernet0/0/1

ip address 192.168.3.1 255.255.255.0

negotiation auto

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:3::1/64

ipv6 enable

ipv6 eigrp 100

interface GigabitEthernet0/1/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/1/1

no ip address

shutdown

negotiation auto

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

interface Vlan1

no ip address

shutdown

router eigrp 100

default-metric 10000 100 255 1 1500

network 10.0.0.0

network 192.168.3.0

redistribute bgp 65002 metric 10000 100 255 1 1500

redistribute ospf 1

redistribute connected metric 10000 100 255 1 1500

eigrp router-id 3.3.3.3

router bgp 65002

bgp log-neighbor-changes

no bgp default ipv4-unicast

neighbor 2001:DB8:ACAD:2::1 remote-as 65001

neighbor 192.168.2.1 remote-as 65001

neighbor 192.168.4.2 remote-as 65003

address-family ipv4

redistribute eigrp 100

neighbor 192.168.2.1 activate

neighbor 192.168.4.2 activate

exit-address-family

address-family ipv6

redistribute eigrp 100 include-connected

neighbor 2001:DB8:ACAD:2::1 activate

exit-address-family

ip forward-protocol nd

no ip http server

no ip http secure-server

ip tftp source-interface GigabitEthernet0

ipv6 router eigrp 100

eigrp router-id 3.3.3.3

redistribute bgp 65002 metric 10000 100 255 1 1500

redistribute connected

control-plane

line con 0

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

_________________________________________________________________________________________________________

Router 4:

Building configuration...

*Nov  4 19:42:54.498: %SYS-5-CONFIG_I: Configured from console by console

Current configuration : 7494 bytes

Last configuration change at 19:42:54 UTC Tue Nov 4 2025

version 17.6

service timestamps debug datetime msec

service timestamps log datetime msec

Call-home is enabled by Smart-Licensing.

service call-home

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R4

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

ipv6 unicast-routing

subscriber templating

vtp domain cisco

vtp mode transparent

multilink bundle-name authenticated

crypto pki trustpoint TP-self-signed-356974628

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-356974628

revocation-check none

rsakeypair TP-self-signed-356974628

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

crypto pki certificate chain TP-self-signed-356974628

certificate self-signed 01

3082032E 30820216 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

30312E30 2C060355 04031325 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 33353639 37343632 38301E17 0D323531 30323731 39303931

355A170D 33353130 32373139 30393135 5A303031 2E302C06 03550403 1325494F

532D5365 6C662D53 69676E65 642D4365 72746966 69636174 652D3335 36393734

36323830 82012230 0D06092A 864886F7 0D010101 05000382 010F0030 82010A02

82010100 C2EAD475 F4B45EB4 638F53E4 CA3C83B1 30140786 2DE0B041 9AA7C212

3633DC46 27524C37 D2990C7A 8B709ED4 B36B7FDE 38E18AD8 91063D97 833D598C

6DAC7259 66986611 8A2A7FBF 85F878FE 0DA5E87C 0F5559D2 EAB19068 3F4C8B8D

40B8B66B 1409D6EB 04261FB9 EF7BD9A9 44757C80 BA18D21E E9A1C266 7361C0F2

7BD90096 9DAE1D21 2EF068E8 DFA2DFB3 51C9D2B1 859314BE 1E35A35E 5BC33F87

FCF0982B 12026AED E2B827C2 E11488AB 067390FA A3222570 5AE3DBEE 6DFBE296

F56714B6 4DB224BA 98AC9846 8B025A19 04527DF2 6FAEC452 F39EFE2F 65304CA4

7517089E 0051FCB7 BD914A61 F059D2A5 975FD5FE 7E9723A7 65731DCE 9A1B318C

A7F4B3C7 02030100 01A35330 51300F06 03551D13 0101FF04 05300301 01FF301F

0603551D 23041830 16801425 1BC5831D 61C8A398 36AAF4F2 4EEAA718 ECEC4630

1D060355 1D0E0416 0414251B C5831D61 C8A39836 AAF4F24E EAA718EC EC46300D

06092A86 4886F70D 01010505 00038201 010085F2 6B83D841 A8F418BF 5EE98F8B

CCA7CAE6 071C550A 64E31E62 46B01444 3EB49C51 0846B3EB 2CED009F 29E0A4F3

252D9604 51E42C25 E374B856 53D77FE5 5A5ACFE6 518698E7 AAA97F3D DFBCBEBC

6C1D48E0 EC5CFD4B 9E12BBA1 0D0AFA4A 516F1D52 354510DF 8FA9933A 3486FBE1

97B8960E B5A10205 A21817FC 92D1A4FD 38B29AFC 256EC163 DD339011 2781C309

B4CFB907 4A8C2EC2 8D9E5208 C7CF4D6E 9D6C0881 36656378 EF77423D 55FBCFEC

2188D24E 9D6E5949 36990449 02D52DE3 FCB6E219 4F28F3E4 0DBCE782 B779AB36

64E6FB96 53471B11 7066B96D 88744EAC E1D13EA1 D4BFF5A9 5C869FD1 52138E15

03C977F8 781AC0BE 98B54D68 F22BE00F 246E

quit

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

30820321 30820209 A0030201 02020101 300D0609 2A864886 F70D0101 0B050030

32310E30 0C060355 040A1305 43697363 6F312030 1E060355 04031317 43697363

6F204C69 63656E73 696E6720 526F6F74 20434130 1E170D31 33303533 30313934

3834375A 170D3338 30353330 31393438 34375A30 32310E30 0C060355 040A1305

43697363 6F312030 1E060355 04031317 43697363 6F204C69 63656E73 696E6720

526F6F74 20434130 82012230 0D06092A 864886F7 0D010101 05000382 010F0030

82010A02 82010100 A6BCBD96 131E05F7 145EA72C 2CD686E6 17222EA1 F1EFF64D

CBB4C798 212AA147 C655D8D7 9471380D 8711441E 1AAF071A 9CAE6388 8A38E520

1C394D78 462EF239 C659F715 B98C0A59 5BBB5CBD 0CFEBEA3 700A8BF7 D8F256EE

4AA4E80D DB6FD1C9 60B1FD18 FFC69C96 6FA68957 A2617DE7 104FDC5F EA2956AC

7390A3EB 2B5436AD C847A2C5 DAB553EB 69A9A535 58E9F3E3 C0BD23CF 58BD7188

68E69491 20F320E7 948E71D7 AE3BCC84 F10684C7 4BC8E00F 539BA42B 42C68BB7

C7479096 B4CB2D62 EA2F505D C7B062A4 6811D95B E8250FC4 5D5D5FB8 8F27D191

C55F0D76 61F9A4CD 3D992327 A8BB03BD 4E6D7069 7CBADF8B DF5F4368 95135E44

DFC7C6CF 04DD7FD1 02030100 01A34230 40300E06 03551D0F 0101FF04 04030201

06300F06 03551D13 0101FF04 05300301 01FF301D 0603551D 0E041604 1449DC85

4B3D31E5 1B3E6A17 606AF333 3D3B4C73 E8300D06 092A8648 86F70D01 010B0500

03820101 00507F24 D3932A66 86025D9F E838AE5C 6D4DF6B0 49631C78 240DA905

604EDCDE FF4FED2B 77FC460E CD636FDB DD44681E 3A5673AB 9093D3B1 6C9E3D8B

D98987BF E40CBD9E 1AECA0C2 2189BB5C 8FA85686 CD98B646 5575B146 8DFC66A8

467A3DF4 4D565700 6ADF0F0D CF835015 3C04FF7C 21E878AC 11BA9CD2 55A9232C

7CA7B7E6 C1AF74F6 152E99B7 B1FCF9BB E973DE7F 5BDDEB86 C71E3B49 1765308B

5FB0DA06 B92AFE7F 494E8A9E 07B85737 F3A58BE1 1A48A229 C37C1E69 39F08678

80DDCD16 D6BACECA EEBC7CF9 8428787B 35202CDC 60E4616A B623CDBD 230E3AFB

418616A9 4093E049 4D10AB75 27E86F73 932E35B5 8862FDAE 0275156F 719BB2F0

D697DF7F 28

quit

license udi pid ISR4321/K9 sn FDO22251WY4

license boot level uck9

memory free low-watermark processor 67522

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 10.10.4.1 255.255.255.0

ipv6 address 2001:DB8:ACAD:14::1/64

ipv6 enable

ipv6 eigrp 100

interface GigabitEthernet0/0/0

ip address 192.168.3.2 255.255.255.0

negotiation auto

ipv6 address FE80::2 link-local

ipv6 address 2001:DB8:ACAD:3::2/64

ipv6 enable

ipv6 eigrp 100

interface GigabitEthernet0/0/1

ip address 192.168.4.1 255.255.255.0

negotiation auto

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:4::1/64

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router eigrp 100

network 10.0.0.0

network 192.168.3.0

network 192.168.4.0

redistribute bgp 65002 metric 100000 100 255 1 1500

redistribute connected metric 10000 100 255 1 1500

eigrp router-id 4.4.4.4

router bgp 65002

bgp log-neighbor-changes

no bgp default ipv4-unicast

neighbor 2001:DB8:ACAD:2::1 remote-as 65001

neighbor 2001:DB8:ACAD:4::2 remote-as 65003

neighbor 192.168.2.1 remote-as 65001

neighbor 192.168.4.2 remote-as 65003

address-family ipv4

redistribute connected

redistribute eigrp 100

neighbor 192.168.2.1 activate

neighbor 192.168.4.2 activate

exit-address-family

address-family ipv6

redistribute connected

redistribute eigrp 100 include-connected

neighbor 2001:DB8:ACAD:2::1 activate

neighbor 2001:DB8:ACAD:4::2 activate

exit-address-family

ip http server

ip http authentication local

ip http secure-server

ip forward-protocol nd

ip tftp source-interface GigabitEthernet0

ipv6 router eigrp 100

eigrp router-id 4.4.4.4

redistribute bgp 65002 metric 10000 100 255 1 1500

redistribute connected metric 10000 100 255 1 1500

control-plane

mgcp behavior rsip-range tgcp-only

mgcp behavior comedia-role none

mgcp behavior comedia-check-media-src disable

mgcp behavior comedia-sdp-force disable

mgcp profile default

line con 0

stopbits 1

line aux 0

line vty 0 4

login

transport input ssh

line vty 5 14

login

transport input ssh

call-home

If contact email address in call-home is configured as sch-smart-licensing@cisco.com

the email address configured in Cisco Smart License Portal will be used as contact email address to send SCH notifications.

contact-email-addr sch-smart-licensing@cisco.com

profile "CiscoTAC-1"

active

destination transport-method http

end

_________________________________________________________________________________________________________

Router 5:

Building configuration...

Current configuration : 4722 bytes

Last configuration change at 18:10:32 UTC Tue Nov 4 2025

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R5

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

no ip domain lookup

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

crypto pki trustpoint TP-self-signed-2949602955

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-2949602955

revocation-check none

rsakeypair TP-self-signed-2949602955

crypto pki certificate chain TP-self-signed-2949602955

certificate self-signed 01

30820330 30820218 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 32393439 36303239 3535301E 170D3235 31303330 31373135

35345A17 0D333030 31303130 30303030 305A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D32 39343936

30323935 35308201 22300D06 092A8648 86F70D01 01010500 0382010F 00308201

0A028201 0100AE0F 762FB968 0847C5A9 9F196D13 016C3690 70682F7A AEDA6F0D

9B06BEAE 4A2110B7 9AC4F91B 16D5616A 2E6633CC 82002D1F EBEC6ADD B17DE844

F8DABBCF 1A044731 C0BCBE47 4599E21F 3E15B18F 40941343 80579C16 0181697F

67F16571 328F81C6 92C0F575 89EF6AA1 E8594755 8ACF8B6D C9D21CF2 5682A929

B41753F6 0023DA12 C6D214CE 500ED734 84F5FEC4 AE1A4775 029CC535 F61B7D11

BA76E204 40920BD0 54C6A9A2 45ACE31F 2B620134 201ED95C 28E1FE47 BD4DBD16

E2808F1C CC9AA7D7 2EBF507F 1A7753F3 EC32DFA4 5642FB01 BD73E9A7 3AF82771

FAC7135D FDC6A799 33347090 11F62DC3 7E811D43 40188876 64B694F1 DC4642C9

E6CF8FF4 5CE30203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF

301F0603 551D2304 18301680 14FDB33B FACAD2BF F1DB78BA 2A5F1862 C0AAE1BA

92301D06 03551D0E 04160414 FDB33BFA CAD2BFF1 DB78BA2A 5F1862C0 AAE1BA92

300D0609 2A864886 F70D0101 05050003 82010100 7FEF380A 46AEA166 4EE4FB94

23ED95BE 519D807A FA751579 63EA158B F1A18075 95E90F31 1B2713CA F6B4C2FA

858C9882 7D00BF48 3888078B 41170FB1 E326749E E93E4D1A FE995A46 508D4C13

30D54496 CB552C88 7B19E665 011039D9 BB6C76CC D1C1D623 D5FCB4DD 4D300613

4F74C271 4BDFAC21 22894703 29FE0626 4C48F438 E2507E56 7E26975B 3B469E7B

46D90540 5FEAB95C 9C1FB3A7 C1A18936 E196E1CA 96554682 4966541F 21368880

25B73BCA 93560134 5BA4E7F8 FF3CCEE6 C73D06FF B9E16825 7E9D062E 6AD9A3B4

BCD1E85D 3EAEF4A5 E97B624D 463DF61B 30B1F1CC 13BFA73A 58C09D60 E3BF0475

F30A1251 CD275E3B FC3289BE 47F79483 E8ABF7D0

quit

license udi pid ISR4321/K9 sn FDO214420HW

license boot level appxk9

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 10.10.5.1 255.255.255.0

ip router isis 1

ipv6 address 2001:DB8:ACAD:15::1/64

ipv6 router isis 1

interface GigabitEthernet0/0/0

ip address 192.168.4.2 255.255.255.0

negotiation auto

ipv6 address FE80::2 link-local

ipv6 address 2001:DB8:ACAD:4::2/64

interface GigabitEthernet0/0/1

ip address 192.168.5.1 255.255.255.0

ip router isis 1

negotiation auto

ipv6 address FE80::1 link-local

ipv6 address 2001:DB8:ACAD:5::1/64

ipv6 router isis 1

interface Serial0/1/0

no ip address

shutdown

interface Serial0/1/1

no ip address

shutdown

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router isis 1

net 49.0001.0000.0000.000a.00

metric-style narrow

redistribute connected

redistribute bgp 65003

address-family ipv6

redistribute connected

redistribute bgp 65003

exit-address-family

router isis area2

metric-style narrow

router bgp 65003

bgp log-neighbor-changes

neighbor 2001:DB8:ACAD:4::1 remote-as 65002

neighbor 192.168.4.1 remote-as 65002

address-family ipv4

redistribute connected

redistribute isis 1 level-1

no neighbor 2001:DB8:ACAD:4::1 activate

neighbor 192.168.4.1 activate

exit-address-family

address-family ipv6

redistribute isis 1 level-1-2 include-connected

redistribute isis level-1

neighbor 2001:DB8:ACAD:4::1 activate

exit-address-family

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

_________________________________________________________________________________________________________

Router 6:

Building configuration...

Current configuration : 3924 bytes

Last configuration change at 18:43:12 UTC Mon Nov 3 2025

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R6

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

no ip domain lookup

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

crypto pki trustpoint TP-self-signed-214699181

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-214699181

revocation-check none

rsakeypair TP-self-signed-214699181

crypto pki certificate chain TP-self-signed-214699181

certificate self-signed 01

3082032E 30820216 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

30312E30 2C060355 04031325 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 32313436 39393138 31301E17 0D323531 30333031 37343032

365A170D 33303031 30313030 30303030 5A303031 2E302C06 03550403 1325494F

532D5365 6C662D53 69676E65 642D4365 72746966 69636174 652D3231 34363939

31383130 82012230 0D06092A 864886F7 0D010101 05000382 010F0030 82010A02

82010100 8B62D3C8 55F0A5FE B0E2CB5F 639D2F4E 6AEDE76B DCAF4916 B8D3B1B4

8159840E D3C5B14E 2F2E0720 B98D2074 F8EC8B9E 7C7A7805 6EE4F264 4EF80D69

5653FEBC A60C70D9 1AAF7550 9F53D107 D51F1687 28931334 EAC362C6 A69D6068

8147A38F D5C3117B 4CE45A51 104D75BC 483B18BE E6B7F7CE 02300C68 E6D187E8

AA6255A1 5AFAC3DE 444C94D3 F63FD3CA 81CE6E90 D65EC55B A1FEB744 B9FA4A48

51B4BF30 60913B40 A30E1782 E68A28ED D27E3362 A215214F 99E1F687 68A1FECA

566C2309 C67005BC 24284EF1 F594DC8B 7DC79763 CF1E6E72 F9C9D7F1 21644E76

CB088EF3 400A85D2 AD896541 9F52CDA7 B6A43F6F F077A623 C600327B 0C13BF3F

A573895B 02030100 01A35330 51300F06 03551D13 0101FF04 05300301 01FF301F

0603551D 23041830 168014E5 47BD6051 0E9C305D 7ED28CDC 6BDD0523 39CFF630

1D060355 1D0E0416 0414E547 BD60510E 9C305D7E D28CDC6B DD052339 CFF6300D

06092A86 4886F70D 01010505 00038201 01003555 82870CE9 E39B082C EC014825

5B41BA17 C9F29B05 3BF80229 62FB5AB8 66BFD4F3 4EB499C6 3EFB598D 9DBA60B0

F77BAC66 9CE11D79 583FABC0 6DACF469 8A9F6E59 078A3513 1BF4EF7B 6AE7F0CB

D6EF07C8 3860938B 90802E9A 8894C7C7 68F042F9 1AEB5F16 00F2D362 B580F570

2007B46F 805DCBF2 CE45EDBB 75DB71AC 83DFDCE8 75EAFECB 874FE410 86E73739

5A01631B 14708C6A D0D658B8 8169BF5F EE5881B5 0257B288 6E0F66E2 ED043055

2B26FAE3 C47DB6CB 1C5421F7 7CEBC265 40CD0A09 C0DC23E3 C00EFF0F 0D904361

2B971042 82596034 38D20FDD D80EEC8A BB23281D A0D94153 25927B5A 50E3CDFB

506E730E 6B705848 979A3B35 C1DB685E D5D3

quit

license udi pid ISR4321/K9 sn FDO21442167

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 10.10.6.1 255.255.255.0

ip router isis 1

ipv6 address 2001:DB8:ACAD:16::1/64

ipv6 router isis 1

interface GigabitEthernet0/0/0

ip address 192.168.5.2 255.255.255.0

ip router isis 1

negotiation auto

ipv6 address FE80::2 link-local

ipv6 address 2001:DB8:ACAD:5::2/64

ipv6 router isis 1

interface GigabitEthernet0/0/1

no ip address

shutdown

negotiation auto

interface Serial0/1/0

no ip address

shutdown

interface Serial0/1/1

no ip address

shutdown

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router isis 1

net 49.0001.0000.0000.000b.00

metric-style narrow

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

_________________________________________________________________________________________________________

Lab Tests:

Pinging across network:

Router 1 ---------> Router 6

R1#ping 10.10.6.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.10.6.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/3/13 ms

R1#traceroute 10.10.6.1

Type escape sequence to abort.

Tracing the route to 10.10.6.1

VRF info: (vrf in name/id, vrf out name/id)

1 192.168.1.2 1 msec 1 msec 1 msec

2 192.168.2.2 1 msec 0 msec 1 msec

3 192.168.3.2 1 msec 1 msec 1 msec

4 192.168.4.2 1 msec 1 msec 1 msec

5 192.168.5.2 2 msec 2 msec *

R1#ping 2001:db8:acad:16::1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:ACAD:16::1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/2/6 ms

R1#traceroute 2001:db8:acad:16::1

Type escape sequence to abort.

Tracing the route to 2001:DB8:ACAD:16::1

1 2001:DB8:ACAD:1::2 2 msec 1 msec 0 msec

2 2001:DB8:ACAD:2::2 0 msec 1 msec 1 msec

3 2001:DB8:ACAD:3::2 1 msec 2 msec 1 msec

4 2001:DB8:ACAD:4::2 1 msec 2 msec 1 msec

5 2001:DB8:ACAD:5::2 1 msec 2 msec 1 msec

Router 6 -------------> Router 1

R6#ping 10.10.1.1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 10.10.1.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/1/2 ms

R6#traceroute 10.10.1.1

Type escape sequence to abort.

Tracing the route to 10.10.1.1

VRF info: (vrf in name/id, vrf out name/id)

1 192.168.5.1 1 msec 0 msec 1 msec

2 192.168.4.1 1 msec 1 msec 1 msec

3 192.168.3.1 0 msec 1 msec 1 msec

4 192.168.2.1 1 msec 2 msec 1 msec

5 192.168.1.1 1 msec *  1 msec

R6#ping 2001:db8:acad:11::1

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 2001:DB8:ACAD:11::1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 1/2/6 ms

R6#traceroute 2001:db8:acad:11::1

Type escape sequence to abort.

Tracing the route to 2001:DB8:ACAD:11::1

1 2001:DB8:ACAD:5::1 1 msec 0 msec 0 msec

2 2001:DB8:ACAD:4::1 1 msec 1 msec 1 msec

3 2001:DB8:ACAD:3::1 1 msec 1 msec 1 msec

4 2001:DB8:ACAD:2::1 1 msec 2 msec 1 msec

5 2001:DB8:ACAD:1::1 1 msec 1 msec 2 msec

OSPF:

R1#show ip ospf neighbor

Neighbor ID     Pri   State           Dead Time   Address         Interface

2.2.2.2           1   FULL/DR         00:00:31    192.168.1.2     GigabitEthernet0/0/1

R1#show ipv6 ospf neighbor

OSPFv3 Router with ID (1.1.1.1) (Process ID 1)

Neighbor ID     Pri   State           Dead Time   Interface ID    Interface

2.2.2.2           1   FULL/DR         00:00:32    6               GigabitEthernet0/0/1

R1#

R2#show ip ospf neighbor

Neighbor ID     Pri   State           Dead Time   Address         Interface

1.1.1.1           1   FULL/BDR        00:00:31    192.168.1.1     GigabitEthernet0/0/0

R2#show ipv6 ospf neighbor

OSPFv3 Router with ID (2.2.2.2) (Process ID 1)

Neighbor ID     Pri   State           Dead Time   Interface ID    Interface

1.1.1.1           1   FULL/BDR        00:00:32    7               GigabitEthernet0/0/0

R2#show ip ospf database

OSPF Router with ID (2.2.2.2) (Process ID 1)

Router Link States (Area 0)

Link ID         ADV Router      Age         Seq#       Checksum Link count

1.1.1.1         1.1.1.1         983         0x8000014B 0x00FD09 1

2.2.2.2         2.2.2.2         974         0x800000B7 0x00EFA0 1

Net Link States (Area 0)

Link ID         ADV Router      Age         Seq#       Checksum

192.168.1.2     2.2.2.2         990         0x800000B0 0x00A960

Summary Net Link States (Area 0)

Link ID         ADV Router      Age         Seq#       Checksum

10.10.1.1       1.1.1.1         1031        0x800000B0 0x00066C

10.10.2.1       2.2.2.2         1030        0x80000132 0x00D614

Router Link States (Area 1)

Link ID         ADV Router      Age         Seq#       Checksum Link count

2.2.2.2         2.2.2.2         1030        0x800000B2 0x00BA9B 1

Summary Net Link States (Area 1)

Link ID         ADV Router      Age         Seq#       Checksum

10.10.1.1       2.2.2.2         990         0x80000001 0x0051CB

192.168.1.0     2.2.2.2         1030        0x800000B0 0x003BDE

Type-5 AS External Link States

Link ID         ADV Router      Age         Seq#       Checksum Tag

10.10.3.0       2.2.2.2         145         0x800000B0 0x00D328 65002

10.10.4.0       2.2.2.2         145         0x80000107 0x00198A 65002

10.10.5.0       2.2.2.2         402         0x8000002C 0x00C6B7 65002

10.10.6.0       2.2.2.2         402         0x8000002C 0x00BBC1 65002

192.168.2.0     2.2.2.2         145         0x80000132 0x007E7C 0

192.168.3.0     2.2.2.2         145         0x80000132 0x00170D 65002

192.168.4.0     2.2.2.2         402         0x8000002C 0x001B0F 65002

192.168.5.0     2.2.2.2         402         0x8000002C 0x001019 65002

R2#show ipv6 ospf database

OSPFv3 Router with ID (2.2.2.2) (Process ID 1)

Router Link States (Area 0)

ADV Router       Age         Seq#        Fragment ID  Link count  Bits

1.1.1.1         1267        0x8000014C  0            1           B

2.2.2.2         1539        0x800000BC  0            1           B E

Net Link States (Area 0)

ADV Router       Age         Seq#        Link ID    Rtr count

2.2.2.2         1556        0x80000001  6          2

Inter Area Prefix Link States (Area 0)

ADV Router       Age         Seq#        Prefix

1.1.1.1         1267        0x800000B0  2001:DB8:ACAD:11::1/128

2.2.2.2         1130        0x800000B1  2001:DB8:ACAD:12::1/128

Link (Type-8) Link States (Area 0)

ADV Router       Age         Seq#        Link ID    Interface

1.1.1.1         1267        0x800000B2  7          Gi0/0/0

2.2.2.2         1130        0x800000B3  6          Gi0/0/0

Intra Area Prefix Link States (Area 0)

ADV Router       Age         Seq#        Link ID    Ref-lstype  Ref-LSID

2.2.2.2         1556        0x80000001  6144       0x2002      6

Router Link States (Area 1)

ADV Router       Age         Seq#        Fragment ID  Link count  Bits

2.2.2.2         1596        0x800000B3  0            0           B E

Inter Area Prefix Link States (Area 1)

ADV Router       Age         Seq#        Prefix

2.2.2.2         1130        0x800000B0  2001:DB8:ACAD:1::/64

2.2.2.2         1556        0x80000001  2001:DB8:ACAD:11::1/128

Intra Area Prefix Link States (Area 1)

ADV Router       Age         Seq#        Link ID    Ref-lstype  Ref-LSID

2.2.2.2         1130        0x800000AF  0          0x2001      0

Type-5 AS External Link States

ADV Router       Age         Seq#        Prefix

2.2.2.2         625         0x80000002  2001:DB8:ACAD:3::/64

2.2.2.2         625         0x80000002  2001:DB8:ACAD:5::/64

2.2.2.2         625         0x80000002  2001:DB8:ACAD:13::/64

2.2.2.2         625         0x80000002  2001:DB8:ACAD:14::/64

2.2.2.2         625         0x80000002  2001:DB8:ACAD:15::/64

2.2.2.2         625         0x80000002  2001:DB8:ACAD:16::/64

2.2.2.2         368         0x80000002  2001:DB8:ACAD:4::/64

R2#show ip route ospf

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

O IA     10.10.1.1/32 [110/2] via 192.168.1.1, 00:16:59, GigabitEthernet0/0/0

R2#show ipv6 route ospf

IPv6 Routing Table - default - 15 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, a - Application

OI  2001:DB8:ACAD:11::1/128 [110/1]

via FE80::1, GigabitEthernet0/0/0

EIGRP:

NEIGHBORS:

R3#show ip eigrp neighbors

EIGRP-IPv4 Neighbors for AS(100)

H   Address                 Interface              Hold Uptime   SRTT   RTO  Q  Seq

(sec)         (ms)       Cnt Num

0   192.168.3.2             Gi0/0/1                  12 1w0d        1   100  0  51

R3#show ipv6 eigrp neighbors

EIGRP-IPv6 Neighbors for AS(100)

H   Address                 Interface              Hold Uptime   SRTT   RTO  Q  Seq

(sec)         (ms)       Cnt Num

0   Link-local address:     Gi0/0/1                  13 1d00h     268  1608  0  17

FE80::2

R4#show ip eigrp neighbors

EIGRP-IPv4 Neighbors for AS(100)

H   Address                 Interface              Hold Uptime   SRTT   RTO  Q  Seq

(sec)         (ms)       Cnt Num

0   192.168.3.1             Gi0/0/0                  11 1w0d        1   100  0  51

R4#show ipv6 eigrp neighbors

EIGRP-IPv6 Neighbors for AS(100)

H   Address                 Interface              Hold Uptime   SRTT   RTO  Q  Seq

(sec)         (ms)       Cnt Num

0   Link-local address:     Gi0/0/0                  13 1d00h       1   100  0  17

FE80::1

R4#show ip eigrp topology

EIGRP-IPv4 Topology Table for AS(100)/ID(4.4.4.4)

Codes: P - Passive, A - Active, U - Update, Q - Query, R - Reply,

r - reply Status, s - sia Status

P 192.168.3.0/24, 1 successors, FD is 2816

via Connected, GigabitEthernet0/0/0

P 10.10.5.0/24, 1 successors, FD is 51200, tag is 65003

via Redistributed (51200/0)

P 192.168.2.0/24, 1 successors, FD is 281856

via 192.168.3.1 (281856/281600), GigabitEthernet0/0/0

P 10.10.1.1/32, 1 successors, FD is 281856, tag is 65001

via 192.168.3.1 (281856/281600), GigabitEthernet0/0/0

P 10.10.4.0/24, 1 successors, FD is 128256

via Connected, Loopback0

P 192.168.1.0/24, 1 successors, FD is 281856, tag is 65001

via 192.168.3.1 (281856/281600), GigabitEthernet0/0/0

P 10.10.6.0/24, 1 successors, FD is 51200, tag is 65003

via Redistributed (51200/0)

P 192.168.4.0/24, 1 successors, FD is 2816

via Connected, GigabitEthernet0/0/1

P 192.168.5.0/24, 1 successors, FD is 51200, tag is 65003

via Redistributed (51200/0)

P 10.10.2.0/24, 1 successors, FD is 281856, tag is 65001

via 192.168.3.1 (281856/281600), GigabitEthernet0/0/0

P 10.10.3.0/24, 1 successors, FD is 130816

via 192.168.3.1 (130816/128256), GigabitEthernet0/0/0

R3#show ip route eigrp

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

D        10.10.4.0/24 [90/130816] via 192.168.3.2, 6d02h, GigabitEthernet0/0/1

D EX     10.10.5.0/24 [170/51456] via 192.168.3.2, 1d00h, GigabitEthernet0/0/1

D EX     10.10.6.0/24 [170/51456] via 192.168.3.2, 1d00h, GigabitEthernet0/0/1

D     192.168.4.0/24 [90/3072] via 192.168.3.2, 1d00h, GigabitEthernet0/0/1

D EX  192.168.5.0/24 [170/51456] via 192.168.3.2, 1d00h, GigabitEthernet0/0/1

R3#show ipv6 route eigrp

IPv6 Routing Table - default - 15 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, a - Application

EX  2001:DB8:ACAD:4::/64 [170/281856]

via FE80::2, GigabitEthernet0/0/1

EX  2001:DB8:ACAD:5::/64 [170/281856], tag 65003

via FE80::2, GigabitEthernet0/0/1

D   2001:DB8:ACAD:14::/64 [90/130816]

via FE80::2, GigabitEthernet0/0/1

EX  2001:DB8:ACAD:15::/64 [170/281856], tag 65003

via FE80::2, GigabitEthernet0/0/1

EX  2001:DB8:ACAD:16::/64 [170/281856], tag 65003

via FE80::2, GigabitEthernet0/0/1

R4#show ip route eigrp

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, m - OMP

n - NAT, Ni - NAT inside, No - NAT outside, Nd - NAT DIA

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

H - NHRP, G - NHRP registered, g - NHRP registration summary

o - ODR, P - periodic downloaded static route, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

& - replicated local route overrides by connected

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

D EX     10.10.1.1/32

[170/281856] via 192.168.3.1, 00:20:36, GigabitEthernet0/0/0

D EX     10.10.2.0/24

[170/281856] via 192.168.3.1, 4d01h, GigabitEthernet0/0/0

D        10.10.3.0/24 [90/130816] via 192.168.3.1, 4d02h, GigabitEthernet0/0/0

D EX  192.168.1.0/24

[170/281856] via 192.168.3.1, 00:21:07, GigabitEthernet0/0/0

D EX  192.168.2.0/24 [170/281856] via 192.168.3.1, 4d01h, GigabitEthernet0/0/0

R4#show ipv6 route eigrp

IPv6 Routing Table - default - 15 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, H - NHRP, I1 - ISIS L1

I2 - ISIS L2, IA - ISIS interarea, IS - ISIS summary, D - EIGRP

EX - EIGRP external, ND - ND Default, NDp - ND Prefix, DCE - Destination

NDr - Redirect, O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1

OE2 - OSPF ext 2, ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2

a - Application, m - OMP

EX  2001:DB8:ACAD:1::/64 [170/281856]

via FE80::1, GigabitEthernet0/0/0

EX  2001:DB8:ACAD:2::/64 [170/3072]

via FE80::1, GigabitEthernet0/0/0

EX  2001:DB8:ACAD:11::1/128 [170/281856]

via FE80::1, GigabitEthernet0/0/0

EX  2001:DB8:ACAD:12::/64 [170/281856]

via FE80::1, GigabitEthernet0/0/0

D   2001:DB8:ACAD:13::/64 [90/130816]

via FE80::1, GigabitEthernet0/0/0

IS-IS:

R5#show clns neighbors

Tag 1:

System Id       Interface     SNPA                State  Holdtime  Type Protocol

R6              Gi0/0/1       0027.90d5.fa40      Up     28        L1L2 IS-IS

Tag area2:

R6#show clns neighbors

Tag 1:

System Id       Interface     SNPA                State  Holdtime  Type Protocol

R5              Gi0/0/0       0027.90d5.fad1      Up     9         L1L2 IS-IS

R5#show ip route isis

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

i L1     10.10.6.0/24 [115/20] via 192.168.5.2, 1d01h, GigabitEthernet0/0/1

R5#show ipv6 route isis

IPv6 Routing Table - default - 15 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, la - LISP alt

lr - LISP site-registrations, ld - LISP dyn-eid, lA - LISP away

le - LISP extranet-policy, a - Application

I1  2001:DB8:ACAD:16::/64 [115/20]

via FE80::2, GigabitEthernet0/0/1

R6#show ip route isis

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

i L2     10.10.1.1/32 [115/10] via 192.168.5.1, 00:30:12, GigabitEthernet0/0/0

i L2     10.10.2.0/24 [115/10] via 192.168.5.1, 1d00h, GigabitEthernet0/0/0

i L2     10.10.3.0/24 [115/10] via 192.168.5.1, 1d00h, GigabitEthernet0/0/0

i L2     10.10.4.0/24 [115/10] via 192.168.5.1, 1d00h, GigabitEthernet0/0/0

i L1     10.10.5.0/24 [115/20] via 192.168.5.1, 1d01h, GigabitEthernet0/0/0

i L2  192.168.1.0/24 [115/10] via 192.168.5.1, 00:30:43, GigabitEthernet0/0/0

i L2  192.168.2.0/24 [115/10] via 192.168.5.1, 1d00h, GigabitEthernet0/0/0

i L2  192.168.3.0/24 [115/10] via 192.168.5.1, 1d00h, GigabitEthernet0/0/0

i L2  192.168.4.0/24 [115/10] via 192.168.5.1, 1d00h, GigabitEthernet0/0/0

R6#show ipv6 route isis

IPv6 Routing Table - default - 14 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, a - Application

I2  2001:DB8:ACAD:1::/64 [115/10]

via FE80::1, GigabitEthernet0/0/0

I2  2001:DB8:ACAD:2::/64 [115/10]

via FE80::1, GigabitEthernet0/0/0

I2  2001:DB8:ACAD:3::/64 [115/10]

via FE80::1, GigabitEthernet0/0/0

I2  2001:DB8:ACAD:4::/64 [115/10]

via FE80::1, GigabitEthernet0/0/0

I2  2001:DB8:ACAD:11::1/128 [115/10]

via FE80::1, GigabitEthernet0/0/0

I2  2001:DB8:ACAD:12::/64 [115/10]

via FE80::1, GigabitEthernet0/0/0

I2  2001:DB8:ACAD:13::/64 [115/10]

via FE80::1, GigabitEthernet0/0/0

I2  2001:DB8:ACAD:14::/64 [115/10]

via FE80::1, GigabitEthernet0/0/0

I1  2001:DB8:ACAD:15::/64 [115/20]

via FE80::1, GigabitEthernet0/0/0

BGP:

R2#show ip bgp summary

BGP router identifier 10.10.2.1, local AS number 65001

BGP table version is 36, main routing table version 36

11 network entries using 2728 bytes of memory

11 path entries using 1496 bytes of memory

7/7 BGP path/bestpath attribute entries using 1960 bytes of memory

1 BGP AS-PATH entries using 24 bytes of memory

0 BGP route-map cache entries using 0 bytes of memory

0 BGP filter-list cache entries using 0 bytes of memory

BGP using 6208 total bytes of memory

BGP activity 32/10 prefixes, 36/14 paths, scan interval 60 secs

Neighbor        V           AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

192.168.2.2     4        65002    6471    6476       36    0    0 4d02h           7

R2#show bgp ipv6 summary

% Command accepted but obsolete, unreleased or unsupported; see documentation.

BGP router identifier 10.10.2.1, local AS number 65001

BGP table version is 32, main routing table version 32

11 network entries using 2992 bytes of memory

11 path entries using 1672 bytes of memory

5/5 BGP path/bestpath attribute entries using 1400 bytes of memory

1 BGP AS-PATH entries using 24 bytes of memory

0 BGP route-map cache entries using 0 bytes of memory

0 BGP filter-list cache entries using 0 bytes of memory

BGP using 6088 total bytes of memory

BGP activity 32/10 prefixes, 36/14 paths, scan interval 60 secs

Neighbor        V           AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

2001:DB8:ACAD:2::2

4        65002    6487    6479       32    0    0 4d02h           7

% NOTE: This command is deprecated. Please use 'show bgp ipv6 unicast'

R3#show ip bgp summary

BGP router identifier 192.168.2.2, local AS number 65002

BGP table version is 67, main routing table version 67

11 network entries using 2728 bytes of memory

11 path entries using 1320 bytes of memory

7/7 BGP path/bestpath attribute entries using 1736 bytes of memory

1 BGP AS-PATH entries using 24 bytes of memory

0 BGP route-map cache entries using 0 bytes of memory

0 BGP filter-list cache entries using 0 bytes of memory

BGP using 5808 total bytes of memory

BGP activity 45/23 prefixes, 50/28 paths, scan interval 60 secs

Neighbor        V           AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

192.168.2.1     4        65001    6479    6474       67    0    0 4d02h           4

192.168.4.2     4        65003       0       0        1    0    0 never    Idle

R3#show bgp ipv6 unicast

BGP table version is 503, local router ID is 192.168.2.2

Status codes: s suppressed, d damped, h history, * valid, > best, i - internal,

r RIB-failure, S Stale, m multipath, b backup-path, f RT-Filter,

x best-external, a additional-path, c RIB-compressed,

Origin codes: i - IGP, e - EGP, ? - incomplete

RPKI validation codes: V valid, I invalid, N Not found

Network          Next Hop            Metric LocPrf Weight Path

*>  2001:DB8:ACAD:1::/64

2001:DB8:ACAD:2::1

0             0 65001 ?

r>  2001:DB8:ACAD:2::/64

2001:DB8:ACAD:2::1

0             0 65001 ?

*>  2001:DB8:ACAD:3::/64

::                       0         32768 ?

*>  2001:DB8:ACAD:4::/64

FE80::2             281856         32768 ?

*>  2001:DB8:ACAD:5::/64

FE80::2             281856         32768 ?

*>  2001:DB8:ACAD:11::1/128

2001:DB8:ACAD:2::1

Network          Next Hop            Metric LocPrf Weight Path

1             0 65001 ?

*>  2001:DB8:ACAD:12::/64

2001:DB8:ACAD:2::1

0             0 65001 ?

*>  2001:DB8:ACAD:13::/64

::                       0         32768 ?

*>  2001:DB8:ACAD:14::/64

FE80::2             130816         32768 ?

*>  2001:DB8:ACAD:15::/64

FE80::2             281856         32768 ?

*>  2001:DB8:ACAD:16::/64

FE80::2             281856         32768 ?

R4#show ip bgp summary

BGP router identifier 192.168.3.2, local AS number 65002

BGP table version is 74, main routing table version 74

11 network entries using 2728 bytes of memory

12 path entries using 1632 bytes of memory

5/5 BGP path/bestpath attribute entries using 1480 bytes of memory

1 BGP AS-PATH entries using 24 bytes of memory

0 BGP route-map cache entries using 0 bytes of memory

0 BGP filter-list cache entries using 0 bytes of memory

BGP using 5864 total bytes of memory

BGP activity 50/28 prefixes, 66/43 paths, scan interval 60 secs

11 networks peaked at 18:22:45 Oct 30 2025 UTC (5d02h ago)

Neighbor        V           AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

192.168.2.1     4        65001       0       0        1    0    0 never    Idle

192.168.4.2     4        65003    1616    1624       74    0    0 1d00h           4

R4#show bgp ipv6 unicast

BGP table version is 30, local router ID is 192.168.3.2

Status codes: s suppressed, d damped, h history, * valid, > best, i - internal,

r RIB-failure, S Stale, m multipath, b backup-path, f RT-Filter,

x best-external, a additional-path, c RIB-compressed,

t secondary path, L long-lived-stale,

Origin codes: i - IGP, e - EGP, ? - incomplete

RPKI validation codes: V valid, I invalid, N Not found

Network          Next Hop            Metric LocPrf Weight Path

*>   2001:DB8:ACAD:1::/64

FE80::1             281856         32768 ?

*>   2001:DB8:ACAD:2::/64

FE80::1               3072         32768 ?

*>   2001:DB8:ACAD:3::/64

::                       0         32768 ?

*>   2001:DB8:ACAD:4::/64

::                       0         32768 ?

*>   2001:DB8:ACAD:5::/64

2001:DB8:ACAD:4::2

0             0 65003 ?

*>   2001:DB8:ACAD:11::1/128

FE80::1             281856         32768 ?

R5#show ip bgp summary

BGP router identifier 10.10.5.1, local AS number 65003

BGP table version is 84, main routing table version 84

11 network entries using 2728 bytes of memory

12 path entries using 1632 bytes of memory

5/5 BGP path/bestpath attribute entries using 1400 bytes of memory

1 BGP AS-PATH entries using 24 bytes of memory

0 BGP route-map cache entries using 0 bytes of memory

0 BGP filter-list cache entries using 0 bytes of memory

BGP using 5784 total bytes of memory

BGP activity 54/32 prefixes, 63/40 paths, scan interval 60 secs

Neighbor        V           AS MsgRcvd MsgSent   TblVer  InQ OutQ Up/Down  State/PfxRcd

192.168.4.1     4        65002    1625    1618       84    0    0 1d00h           8

R5#show bgp ipv6 unicast

BGP table version is 18, local router ID is 10.10.5.1

Status codes: s suppressed, d damped, h history, * valid, > best, i - internal,

r RIB-failure, S Stale, m multipath, b backup-path, f RT-Filter,

x best-external, a additional-path, c RIB-compressed,

t secondary path, L long-lived-stale,

Origin codes: i - IGP, e - EGP, ? - incomplete

RPKI validation codes: V valid, I invalid, N Not found

Network          Next Hop            Metric LocPrf Weight Path

*>   2001:DB8:ACAD:1::/64

2001:DB8:ACAD:4::1

281856             0 65002 ?

*>   2001:DB8:ACAD:2::/64

2001:DB8:ACAD:4::1

3072             0 65002 ?

*>   2001:DB8:ACAD:3::/64

2001:DB8:ACAD:4::1

0             0 65002 ?

r>   2001:DB8:ACAD:4::/64

2001:DB8:ACAD:4::1

0             0 65002 ?

*>   2001:DB8:ACAD:5::/64

Network          Next Hop            Metric LocPrf Weight Path

::                       0         32768 ?

*>   2001:DB8:ACAD:11::1/128

2001:DB8:ACAD:4::1

281856             0 65002 ?

*>   2001:DB8:ACAD:12::/64

2001:DB8:ACAD:4::1

281856             0 65002 ?

*>   2001:DB8:ACAD:13::/64

2001:DB8:ACAD:4::1

130816             0 65002 ?

*>   2001:DB8:ACAD:14::/64

2001:DB8:ACAD:4::1

0             0 65002 ?

*>   2001:DB8:ACAD:15::/64

::                       0         32768 ?

*>   2001:DB8:ACAD:16::/64

FE80::2                 20         32768 ?

R3#show ip bgp

BGP table version is 67, local router ID is 192.168.2.2

Status codes: s suppressed, d damped, h history, * valid, > best, i - internal,

r RIB-failure, S Stale, m multipath, b backup-path, f RT-Filter,

x best-external, a additional-path, c RIB-compressed,

Origin codes: i - IGP, e - EGP, ? - incomplete

RPKI validation codes: V valid, I invalid, N Not found

Network          Next Hop            Metric LocPrf Weight Path

*>  10.10.1.1/32     192.168.2.1              2             0 65001 ?

*>  10.10.2.0/24     192.168.2.1              0             0 65001 ?

*>  10.10.3.0/24     0.0.0.0                  0         32768 ?

*>  10.10.4.0/24     192.168.3.2         130816         32768 ?

*>  10.10.5.0/24     192.168.3.2          51456         32768 ?

*>  10.10.6.0/24     192.168.3.2          51456         32768 ?

*>  192.168.1.0      192.168.2.1              0             0 65001 ?

r>  192.168.2.0      192.168.2.1              0             0 65001 i

*>  192.168.3.0      0.0.0.0                  0         32768 ?

*>  192.168.4.0      192.168.3.2           3072         32768 ?

*>  192.168.5.0      192.168.3.2          51456         32768 ?

R4#show ip bgp

BGP table version is 74, local router ID is 192.168.3.2

Status codes: s suppressed, d damped, h history, * valid, > best, i - internal,

r RIB-failure, S Stale, m multipath, b backup-path, f RT-Filter,

x best-external, a additional-path, c RIB-compressed,

t secondary path, L long-lived-stale,

Origin codes: i - IGP, e - EGP, ? - incomplete

RPKI validation codes: V valid, I invalid, N Not found

Network          Next Hop            Metric LocPrf Weight Path

*>   10.10.1.1/32     192.168.3.1         281856         32768 ?

*>   10.10.2.0/24     192.168.3.1         281856         32768 ?

*>   10.10.3.0/24     192.168.3.1         130816         32768 ?

*>   10.10.4.0/24     0.0.0.0                  0         32768 ?

*>   10.10.5.0/24     192.168.4.2              0             0 65003 ?

*>   10.10.6.0/24     192.168.4.2             20             0 65003 ?

*>   192.168.1.0      192.168.3.1         281856         32768 ?

*>   192.168.2.0      192.168.3.1         281856         32768 ?

*>   192.168.3.0      0.0.0.0                  0         32768 ?

*    192.168.4.0      192.168.4.2              0             0 65003 ?

*>                    0.0.0.0                  0         32768 ?

*>   192.168.5.0      192.168.4.2              0             0 65003 ?

R3#show ip bgp

BGP table version is 67, local router ID is 192.168.2.2

Status codes: s suppressed, d damped, h history, * valid, > best, i - internal,

r RIB-failure, S Stale, m multipath, b backup-path, f RT-Filter,

x best-external, a additional-path, c RIB-compressed,

Origin codes: i - IGP, e - EGP, ? - incomplete

RPKI validation codes: V valid, I invalid, N Not found

Network          Next Hop            Metric LocPrf Weight Path

*>  10.10.1.1/32     192.168.2.1              2             0 65001 ?

*>  10.10.2.0/24     192.168.2.1              0             0 65001 ?

*>  10.10.3.0/24     0.0.0.0                  0         32768 ?

*>  10.10.4.0/24     192.168.3.2         130816         32768 ?

*>  10.10.5.0/24     192.168.3.2          51456         32768 ?

*>  10.10.6.0/24     192.168.3.2          51456         32768 ?

*>  192.168.1.0      192.168.2.1              0             0 65001 ?

r>  192.168.2.0      192.168.2.1              0             0 65001 i

*>  192.168.3.0      0.0.0.0                  0         32768 ?

*>  192.168.4.0      192.168.3.2           3072         32768 ?

*>  192.168.5.0      192.168.3.2          51456         32768 ?

R3#show ip route bgp

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

B        10.10.1.1/32 [20/2] via 192.168.2.1, 00:42:25

B        10.10.2.0/24 [20/0] via 192.168.2.1, 4d02h

B     192.168.1.0/24 [20/0] via 192.168.2.1, 00:42:55

R4#show ip route bgp

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, m - OMP

n - NAT, Ni - NAT inside, No - NAT outside, Nd - NAT DIA

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

H - NHRP, G - NHRP registered, g - NHRP registration summary

o - ODR, P - periodic downloaded static route, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

& - replicated local route overrides by connected

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

B        10.10.5.0/24 [20/0] via 192.168.4.2, 1d00h

B        10.10.6.0/24 [20/20] via 192.168.4.2, 1d00h

B     192.168.5.0/24 [20/0] via 192.168.4.2, 1d00h

R4#show ipv6 route bgp

IPv6 Routing Table - default - 15 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, H - NHRP, I1 - ISIS L1

I2 - ISIS L2, IA - ISIS interarea, IS - ISIS summary, D - EIGRP

EX - EIGRP external, ND - ND Default, NDp - ND Prefix, DCE - Destination

NDr - Redirect, O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1

OE2 - OSPF ext 2, ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2

a - Application, m - OMP

B   2001:DB8:ACAD:5::/64 [20/0], tag 65003

via FE80::2, GigabitEthernet0/0/1

B   2001:DB8:ACAD:15::/64 [20/0], tag 65003

via FE80::2, GigabitEthernet0/0/1

B   2001:DB8:ACAD:16::/64 [20/20], tag 65003

via FE80::2, GigabitEthernet0/0/1

R5#show ip route bgp

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

B        10.10.1.1/32 [20/281856] via 192.168.4.1, 00:42:42

B        10.10.2.0/24 [20/281856] via 192.168.4.1, 1d00h

B        10.10.3.0/24 [20/130816] via 192.168.4.1, 1d00h

B        10.10.4.0/24 [20/0] via 192.168.4.1, 1d00h

B     192.168.1.0/24 [20/281856] via 192.168.4.1, 00:43:13

B     192.168.2.0/24 [20/281856] via 192.168.4.1, 1d00h

B     192.168.3.0/24 [20/0] via 192.168.4.1, 1d00h

Problems:

When implementing IS-IS, we ran into the problem of categorization. The category of IS-IS can be segmented into two levels: Level 1 and Level 2. We initially configured R5 to level 1 and R6 to level 2, resulting in failure when running redistribution commands like redistribute isis level-1. Another problem is also putting our loopback in level-2, causing loopback addresses to be excluded in the process of redistribution. After this, we learned that redistribution works when you are within the same level if you redistribute at that same IS-IS level.

When implementing EIGRP, we didn’t account for the complexity of how EIGRP operates. EIGRP doesn’t utilize the normal path finding or Dijkstra’s algorithm to find their optimal paths. EIGRP uses metrics that rely on bandwidth, delay, reliability, load, and MTU. This means we needed to translate other routing protocol’s costs (like OSPF or IS-IS) and convert to EIGRP. By setting the default metrics with redistribute bgp 65001 include-connected 10000 100 255 1 1500, they can be automatically translated to that default configuration, allowing the autonomous system to use EIGRP to learn BGP routes.

Conclusion:

When interconnecting multiple different routing protocols together, the BGP protocol could be a phenomenal tool that not only introduces connectivity, but its flexible parameters and policy-specific settings can be heavily favored especially in enterprise-level environments. With the right configuration, any complex network can be simplified and linked with the appropriate BGP configuration.